%% ============================================================

% MAIN: King vs DIM (memory-lean) comparison + profiler (time & memory)

% + EXTRA FIGS (FHT only):

% (1) xOz (from King/FHT): |p| and SPL on (x,z)

% (2) xOy @ z≈1 m (from King/FHT): SPL and phase on (x,y)

% via planar phase extension: p(x,y)=p(rho)*exp(j*m*phi)

% ============================================================

clear; clc; close all;

%% -------------------- medium --------------------

medium.c0 = 343;

medium.rho0 = 1.21;

medium.beta = 1.2;

medium.pref = 2e-5;

medium.use_absorp = true;

medium.atten_handle = @(f) AbsorpAttenCoef(f);

%% -------------------- source --------------------

source.profile = 'Vortex-m'; % 'Uniform' | 'Focus' | 'Vortex-m' | 'Poly' | 'Custom'

source.a = 0.1;

source.v0 = 0.172;

source.v_ratio = 1;

source.m = 0;

source.F = 0.2;

source.f1 = 42e3;

source.fa = 2e3;

%% -------------------- calc parameters --------------------

calc = struct();

% --- FHT / King ---

calc.fht.N_FHT = 32768;

calc.fht.rho_max = 1.2;

calc.fht.Nh_scale = 1.2;

calc.fht.NH_scale = 4;

calc.fht.Nh_v_scale = 1.1;

calc.fht.zu_max = 4;

calc.fht.za_max = 2;

% --- DIM source discretization ---

calc.dim.use_freq = 'f2';

calc.dim.dis_coe = 16;

calc.dim.margin = 1;

calc.dim.method = 'rayleigh'; % 'rayleigh' or 'asm'

% --- King analytic spectrum stability ---

calc.king.method = 'analytic'; % 'analytic' or 'transform'

calc.king.eps_kzz = 1e-3; % keep your naming

% map to function-expected fields:

calc.king.eps_phase = calc.king.eps_kzz;

calc.king.kz_min = 1e-12;

% --- ASM settings (used only if calc.dim.method='asm') ---

calc.asm.pad_factor = 16;

calc.asm.kzz_eps = 1e-12;

%% -------------------- target plane --------------------

z_target = 1.0;

ds = 16;

%% ==================== PROFILER: helpers ====================

get_mem_mb = @() local_get_mem_mb(); % returns NaN if not available

fmt_mem = @(x) local_fmt_mem(x);

%% -------------------- King only (to get grids) --------------------

obs_grid_dummy.dim.x = 0;

obs_grid_dummy.dim.y = 0;

obs_grid_dummy.dim.z = 0;

fprintf('\n==================== KING (time & memory) ====================\n');

mem0 = get_mem_mb(); t0 = tic;

resK = calc_ultrasound_field( ...

source, medium, calc, obs_grid_dummy, ...

'king', calc.dim.method, calc.king.method);

tK = toc(t0); mem1 = get_mem_mb();

fprintf('King time: %.3f s\n', tK);

fprintf('King memory: start %s, end %s, delta %s\n', fmt_mem(mem0), fmt_mem(mem1), fmt_mem(mem1-mem0));

rho_full = resK.king.rho(:);

zK = resK.king.z(:);

[~, iz] = min(abs(zK - z_target));

z_use = zK(iz);

rho_ds = rho_full(1:ds:end);

%% -------------------- DIM --------------------

fprintf('\n==================== DIM (time & memory) =====================\n');

mem2 = get_mem_mb(); t1 = tic;

if strcmpi(calc.dim.method,'rayleigh')

% Rayleigh: compute directly on rho_ds

obs_grid.dim.x = rho_ds(:).';

obs_grid.dim.y = 0;

obs_grid.dim.z = z_use;

obs_grid.dim.block_size = 200000;

resD = calc_ultrasound_field( ...

source, medium, calc, obs_grid, ...

'dim', 'rayleigh', calc.king.method);

elseif strcmpi(calc.dim.method,'asm')

% ASM: compute on native uniform grid (function output), then interp in MAIN

obs_grid.dim.x = 0; % not used by ASM computation

obs_grid.dim.y = 0; % not used by ASM computation

obs_grid.dim.z = z_use;

resD = calc_ultrasound_field( ...

source, medium, calc, obs_grid, ...

'dim', 'asm', calc.king.method);

else

error('calc.dim.method must be ''rayleigh'' or ''asm''.');

end

tD = toc(t1); mem3 = get_mem_mb();

fprintf('DIM time: %.3f s\n', tD);

fprintf('DIM memory: start %s, end %s, delta %s\n', fmt_mem(mem2), fmt_mem(mem3), fmt_mem(mem3-mem2));

%% -------------------- extract fields (use f1 here to match your current code) --------------------

pK = resK.king.p_f1(:, iz);

pK = pK(1:ds:end); % compare on rho_ds

if strcmpi(calc.dim.method,'rayleigh')

pD = squeeze(resD.dim.p_f1(1,:,1)).'; % Nx x 1, already on rho_ds

else

% ASM output: p_f1 is Ny x Nx x Nz on native uniform grid

xA = resD.dim.x(:);

yA = resD.dim.y(:);

zA = resD.dim.z(:);

[~, iy0] = min(abs(yA - 0));

[~, iz0] = min(abs(zA - z_use));

pA_line = squeeze(resD.dim.p_f1(iy0,:,iz0)).'; % numel(xA) x 1

% interpolate to rho_ds ONLY in MAIN (as requested)

pD = interp1(xA, pA_line, rho_ds, 'linear', 0);

end

%% -------------------- metrics --------------------

magK = abs(pK);

magD = abs(pD);

phK = unwrap(angle(pK));

phD = unwrap(angle(pD));

eps0 = 1e-12;

rel_err_log = log10( abs(pD - pK) ./ (abs(pK) + eps0) );

%% -------------------- plots: 1D compare (rho line) --------------------

figure('Name',sprintf('King vs DIM-%s (z≈1 m)', upper(string(calc.dim.method))), ...

'position',[100 100 1200 1200]);

subplot(4,1,1);

plot(rho_ds, 20*log10(magK / medium.pref / sqrt(2)), 'LineWidth',1.5); hold on;

plot(rho_ds, 20*log10(magD / medium.pref / sqrt(2)), '--', 'LineWidth',1.5);

grid on;

xlabel('\rho (m)'); ylabel('SPL (dB)');

title(sprintf('Magnitude @ z = %.6f m', z_use));

legend('King','DIM');

subplot(4,1,2);

plot(rho_ds, magK, 'LineWidth',1.5); hold on;

plot(rho_ds, magD, '--', 'LineWidth',1.5);

grid on;

xlabel('\rho (m)'); ylabel('|p| (Pa)');

title(sprintf('Magnitude @ z = %.6f m', z_use));

legend('King','DIM');

subplot(4,1,3);

plot(rho_ds, phK, 'LineWidth',1.5); hold on;

plot(rho_ds, phD, '--', 'LineWidth',1.5);

grid on;

xlabel('\rho (m)'); ylabel('Phase (rad)');

title('Phase (unwrap)');

legend('King','DIM');

subplot(4,1,4);

plot(rho_ds, rel_err_log, 'LineWidth',1.5);

grid on;

xlabel('\rho (m)');

ylabel('log_{10} relative error');

title('log_{10}(|p_{DIM}-p_{King}| / |p_{King}|)');

fprintf('\n==================== SUMMARY ====================\n');

fprintf('dim_method = %s\n', string(calc.dim.method));

fprintf('z_target = %.3f m, z_use = %.6f m (nearest King grid)\n', z_target, z_use);

fprintf('King: %.3f s, DIM: %.3f s\n', tK, tD);

fprintf('NOTE: memory readings may be NaN if OS API is unavailable.\n');

%% ============================================================

%% ============================================================

%% ============================================================

% 2D FIG #1: xOz (FHT/King only)

% - show |p| (AMP) and SPL on xOz

% - each figure uses a single unified colorbar range (trivial, single axes)

% ============================================================

% choose which frequency field to plot

p_rz = resK.king.p_f1; % Nr x Nz

z_ultra = resK.king.z(:).'; % 1 x Nz

r_ultra = resK.king.rho(:); % Nr x 1

rho_max = calc.fht.rho_max;

idx_r = find(r_ultra <= rho_max, 1, 'last');

if isempty(idx_r); idx_r = numel(r_ultra); end

r_use = r_ultra(1:idx_r); % Nr_use x 1

p_use = p_rz(1:idx_r, :); % Nr_use x Nz

% mirror to xOz (no interpolation)

x_axis = [flipud(-r_use); r_use]; % (2*Nr_use) x 1

p_xz = [flipud(p_use); p_use]; % (2*Nr_use) x Nz

AMP_xz = abs(p_xz);

SPL_xz = 20*log10(AMP_xz / medium.pref / sqrt(2)); % RMS SPL

% ---- AMP (|p|) map ----

figure('Name','FHT: xOz AMP (|p|)');

pcolor(z_ultra, x_axis, AMP_xz);

colormap(MyColor('vik'));

shading interp;

clb = colorbar;

clb.Title.Interpreter = 'latex';

clb.Title.String = '$|p|$ (Pa)';

set(clb,'Fontsize',20);

xlim([0 z_ultra(end)]); ylim([-rho_max rho_max]);

fontsize(gca,24,'points');

xlabel('$z$ (m)', 'Interpreter','latex','Fontsize',21);

ylabel('$x$ (m)', 'Interpreter','latex','Fontsize',21);

pbaspect([z_ultra(end), 2*rho_max, 1]);

set(gca,'linewidth',2);

set(gca,'TickLabelInterpreter','latex');

% ---- SPL map ----

figure('Name','FHT: xOz SPL');

pcolor(z_ultra, x_axis, SPL_xz);

colormap(MyColor('vik'));

shading interp;

clb = colorbar;

clb.Title.Interpreter = 'latex';

clb.Title.String = '$\mathrm{SPL}$ (dB re $p_{\mathrm{ref}}$, RMS)';

set(clb,'Fontsize',20);

xlim([0 z_ultra(end)]); ylim([-rho_max rho_max]);

fontsize(gca,24,'points');

xlabel('$z$ (m)', 'Interpreter','latex','Fontsize',21);

ylabel('$x$ (m)', 'Interpreter','latex','Fontsize',21);

pbaspect([z_ultra(end), 2*rho_max, 1]);

set(gca,'linewidth',2);

set(gca,'TickLabelInterpreter','latex');

% optional:

% clim([45,75]);

%% ============================================================

% 2D FIG #2: xOy @ z≈1 m (FHT vs DIM) — SAME colorbar per figure

% - Construct polar grid (rho,theta) and do phase extension:

% p2D(r,theta) = p_line(r) * exp(1j*m*theta)

% - Plot SPL and AMP in separate figures

% - In EACH figure, the two subplots share the SAME clim

% ============================================================

% ---- view settings ----

r_boundary = 0.30; % radius to show (m)

theta_fig = 0:0.01:2*pi;

% ---- m ----

if isfield(resK,'source') && isfield(resK.source,'m_used')

m_use = resK.source.m_used;

else

m_use = source.m;

end

% ---- radial samples (NO interp) ----

rho_fig = rho_ds(:).'; % 1 x Nr_fig

pK_full = resK.king.p_f1(:, iz); % Nr_full x 1

pK_line = pK_full(1:ds:end).'; % 1 x Nr_fig

pD_line = pD(:).'; % 1 x Nr_fig (already on rho_ds)

% crop to r_boundary

idx_rb = find(rho_fig <= r_boundary, 1, 'last');

if isempty(idx_rb); idx_rb = 1; end

rho_fig = rho_fig(1:idx_rb);

pK_line = pK_line(1:idx_rb);

pD_line = pD_line(1:idx_rb);

% polar grid -> Cartesian

[TH, R] = meshgrid(theta_fig, rho_fig);

[X, Y] = pol2cart(TH, R);

% phase extension

pK_2D = (pK_line(:) * ones(1, numel(theta_fig))) .* exp(1i*m_use*TH);

pD_2D = (pD_line(:) * ones(1, numel(theta_fig))) .* exp(1i*m_use*TH);

% fields

AMP_K = abs(pK_2D);

AMP_D = abs(pD_2D);

SPL_K = 20*log10(AMP_K / medium.pref / sqrt(2)); % RMS SPL

SPL_D = 20*log10(AMP_D / medium.pref / sqrt(2));

PH_K = angle(pK_2D)/pi;

PH_D = angle(pD_2D)/pi;

% ---- unified color limits (important) ----

spl_lim = [min([SPL_K(:); SPL_D(:)]), max([SPL_K(:); SPL_D(:)])];

amp_lim = [min([AMP_K(:); AMP_D(:)]), max([AMP_K(:); AMP_D(:)])];

ph_lim = [-1 1];

%% ---- FIG: xOy SPL (two subplots, SAME colorbar range) ----

figure('Name',sprintf('xOy SPL @ z=%.6f m, m=%d (FHT vs DIM)', z_use, m_use), 'position',[100 100 1400 650]);

subplot(1,2,1);

pcolor(X, Y, SPL_K);

shading flat

xlim([-1.2*r_boundary 1.2*r_boundary]);

ylim([-1.2*r_boundary 1.2*r_boundary]);

pbaspect([1 1 1])

colormap(MyColor('vik'))

clim(spl_lim);

clb = colorbar;

clb.Title.Interpreter = 'latex';

clb.Title.String = '$\mathrm{SPL}$ (dB re $p_{\mathrm{ref}}$, RMS)';

set(clb,'Fontsize',18);

set(gca,'position',[0.07 0.12 0.38 0.78]);

set(gca,'linewidth',2);

set(gca,'TickLabelInterpreter','latex');

xlabel('$x$ (m)','Interpreter','latex','Fontsize',18);

ylabel('$y$ (m)','Interpreter','latex','Fontsize',18);

title('FHT (from King)','Interpreter','latex','Fontsize',20);

subplot(1,2,2);

pcolor(X, Y, SPL_D);

shading flat

xlim([-1.2*r_boundary 1.2*r_boundary]);

ylim([-1.2*r_boundary 1.2*r_boundary]);

pbaspect([1 1 1])

colormap(MyColor('vik'))

clim(spl_lim);

clb = colorbar;

clb.Title.Interpreter = 'latex';

clb.Title.String = '$\mathrm{SPL}$ (dB re $p_{\mathrm{ref}}$, RMS)';

set(clb,'Fontsize',18);

set(gca,'position',[0.55 0.12 0.38 0.78]);

set(gca,'linewidth',2);

set(gca,'TickLabelInterpreter','latex');

xlabel('$x$ (m)','Interpreter','latex','Fontsize',18);

ylabel('$y$ (m)','Interpreter','latex','Fontsize',18);

title(sprintf('DIM-%s', upper(string(calc.dim.method))), 'Interpreter','latex','Fontsize',20);

sgtitle(sprintf('$xOy$ SPL @ $z=%.6f$ m, $m=%d$, $r\\le%.2f$ m', z_use, m_use, r_boundary), ...

'Interpreter','latex','Fontsize',20);

%% ---- FIG: xOy AMP (|p|) (two subplots, SAME colorbar range) ----

figure('Name',sprintf('xOy AMP @ z=%.6f m, m=%d (FHT vs DIM)', z_use, m_use), 'position',[100 100 1400 650]);

subplot(1,2,1);

pcolor(X, Y, AMP_K);

shading flat

xlim([-1.2*r_boundary 1.2*r_boundary]);

ylim([-1.2*r_boundary 1.2*r_boundary]);

pbaspect([1 1 1])

colormap(MyColor('vik'))

clim(amp_lim);

clb = colorbar;

clb.Title.Interpreter = 'latex';

clb.Title.String = '$|p|$ (Pa)';

set(clb,'Fontsize',18);

set(gca,'position',[0.07 0.12 0.38 0.78]);

set(gca,'linewidth',2);

set(gca,'TickLabelInterpreter','latex');

xlabel('$x$ (m)','Interpreter','latex','Fontsize',18);

ylabel('$y$ (m)','Interpreter','latex','Fontsize',18);

title('FHT (from King)','Interpreter','latex','Fontsize',20);

subplot(1,2,2);

pcolor(X, Y, AMP_D);

shading flat

xlim([-1.2*r_boundary 1.2*r_boundary]);

ylim([-1.2*r_boundary 1.2*r_boundary]);

pbaspect([1 1 1])

colormap(MyColor('vik'))

clim(amp_lim);

clb = colorbar;

clb.Title.Interpreter = 'latex';

clb.Title.String = '$|p|$ (Pa)';

set(clb,'Fontsize',18);

set(gca,'position',[0.55 0.12 0.38 0.78]);

set(gca,'linewidth',2);

set(gca,'TickLabelInterpreter','latex');

xlabel('$x$ (m)','Interpreter','latex','Fontsize',18);

ylabel('$y$ (m)','Interpreter','latex','Fontsize',18);

title(sprintf('DIM-%s', upper(string(calc.dim.method))), 'Interpreter','latex','Fontsize',20);

sgtitle(sprintf('$xOy$ AMP ($|p|$) @ $z=%.6f$ m, $m=%d$, $r\\le%.2f$ m', z_use, m_use, r_boundary), ...

'Interpreter','latex','Fontsize',20);

%% ---- FIG: xOy Phase/pi (two subplots, SAME colorbar range) ----

figure('Name',sprintf('xOy Phase/pi @ z=%.6f m, m=%d (FHT vs DIM)', z_use, m_use), 'position',[100 100 1400 650]);

subplot(1,2,1);

pcolor(X, Y, PH_K);

shading flat

xlim([-1.2*r_boundary 1.2*r_boundary]);

ylim([-1.2*r_boundary 1.2*r_boundary]);

pbaspect([1 1 1])

colormap('hsv')

clim(ph_lim);

clb = colorbar;

clb.Title.Interpreter = 'latex';

clb.Title.String = '$\angle p/\pi$';

set(clb,'Fontsize',18);

set(gca,'position',[0.07 0.12 0.38 0.78]);

set(gca,'linewidth',2);

set(gca,'TickLabelInterpreter','latex');

xlabel('$x$ (m)','Interpreter','latex','Fontsize',18);

ylabel('$y$ (m)','Interpreter','latex','Fontsize',18);

title('FHT (from King)','Interpreter','latex','Fontsize',20);

subplot(1,2,2);

pcolor(X, Y, PH_D);

shading flat

xlim([-1.2*r_boundary 1.2*r_boundary]);

ylim([-1.2*r_boundary 1.2*r_boundary]);

pbaspect([1 1 1])

colormap('hsv')

clim(ph_lim);

clb = colorbar;

clb.Title.Interpreter = 'latex';

clb.Title.String = '$\angle p/\pi$';

set(clb,'Fontsize',18);

set(gca,'position',[0.55 0.12 0.38 0.78]);

set(gca,'linewidth',2);

set(gca,'TickLabelInterpreter','latex');

xlabel('$x$ (m)','Interpreter','latex','Fontsize',18);

ylabel('$y$ (m)','Interpreter','latex','Fontsize',18);

title(sprintf('DIM-%s', upper(string(calc.dim.method))), 'Interpreter','latex','Fontsize',20);

sgtitle(sprintf('$xOy$ Phase$/\\pi$ @ $z=%.6f$ m, $m=%d$, $r\\le%.2f$ m', z_use, m_use, r_boundary), ...

'Interpreter','latex','Fontsize',20);

%% ============================================================

%% ============================================================

%% ============================================================

%% ============================================================

% VELOCITY FIELD (FHT/King only) on xOy @ z = z_use

% Using: v = -(i/(rho0*omega)) * grad(p)

% With vortex phase extension: p(rho,phi,z)=p0(rho,z)*exp(1j*m*phi)

% Cylindrical components:

% v_r = -(i/(rho0*omega)) * d p / d rho

% v_phi = -(i/(rho0*omega)) * (1/rho) * d p / d phi

% = (m/(rho0*omega*rho)) * p

% v_z = -(i/(rho0*omega)) * d p / d z

% Then convert to Cartesian on xOy:

% v_x = v_r*cos(phi) - v_phi*sin(phi)

% v_y = v_r*sin(phi) + v_phi*cos(phi)

%% ============================================================

% ---------- parameters ----------

rho0 = medium.rho0;

f_use = resK.source.f1; % 你这里画的是 p_f1，就用 f1

omega = 2*pi*f_use;

% topological charge (IMPORTANT: use resK.source)

if isfield(resK,'source') && isfield(resK.source,'m_used')

m_use = resK.source.m_used;

elseif isfield(resK,'source') && isfield(resK.source,'m')

m_use = resK.source.m;

else

m_use = 0;

end

% ---------- get FHT grids & p0(rho,z) ----------

rho_full = resK.king.rho(:); % Nr_full x 1

z_full = resK.king.z(:); % Nz x 1

p0_rz = resK.king.p_f1; % Nr_full x Nz (complex)

% crop to a viewing rho range if desired (optional)

rho_max = calc.fht.rho_max;

idx_rmax = find(rho_full <= rho_max, 1, 'last');

if isempty(idx_rmax), idx_rmax = numel(rho_full); end

rho_full_use = rho_full(1:idx_rmax);

p0_rz_use = p0_rz(1:idx_rmax, :); % Nr_use x Nz

Nr_use = numel(rho_full_use);

Nz = numel(z_full);

% ---------- robust derivatives on (rho,z) grid ----------

% NOTE: some MATLAB versions are picky about gradient spacing vectors.

% Use scalar step sizes (grid is typically uniform enough in your FHT sampling).

dr = mean(diff(rho_full_use));

dz = mean(diff(z_full));

% p0_rz_use is (Nr_use x Nz), but we want matrix as (Nz x Nr_use) for z-first indexing

P_zr = (p0_rz_use.'); % Nz x Nr_use

% Derivatives: d/dz along rows, d/dr along cols

[dPdz_zr, dPdr_zr] = gradient(P_zr, dz, dr); % both Nz x Nr_use

% take the z slice at iz (nearest to z_target already chosen in your main)

% iz corresponds to z_full index

p0_line_full = P_zr(iz, :).'; % Nr_use x 1

dpdr_line = dPdr_zr(iz, :).'; % Nr_use x 1

dpdz_line = dPdz_zr(iz, :).'; % Nr_use x 1

% ---------- downsample to rho_ds (NO interpolation) ----------

% rho_ds is taken as rho_full(1:ds:end) earlier; keep consistent:

rho_ds_full = rho_full(1:ds:end);

% but we cropped to idx_rmax; ensure consistent limit:

rho_ds = rho_ds_full(rho_ds_full <= rho_full_use(end));

% indices on the cropped arrays (since rho_full_use starts at same rho_full(1))

idx_ds = 1:ds:numel(rho_full_use);

idx_ds = idx_ds(1:numel(rho_ds));

p0_line = p0_line_full(idx_ds); % Nds x 1

dpdr = dpdr_line(idx_ds); % Nds x 1

dpdz = dpdz_line(idx_ds); % Nds x 1

% ---------- build xOy polar grid (no interp; like your p extension) ----------

r_boundary = 0.30; % view radius, adjust

theta_fig = 0:0.01:2*pi;

idx_rb = find(rho_ds <= r_boundary, 1, 'last');

if isempty(idx_rb), idx_rb = 1; end

rho_fig = rho_ds(1:idx_rb).'; % 1 x Nr_fig

p0_line = p0_line(1:idx_rb);

dpdr = dpdr(1:idx_rb);

dpdz = dpdz(1:idx_rb);

[TH, R] = meshgrid(theta_fig, rho_fig); % (Nr_fig x Nt)

[X, Y] = pol2cart(TH, R);

% extend pressure

P_2D = (p0_line(:) * ones(1,numel(theta_fig))) .* exp(1j*m_use*TH);

% extend derivatives (dp/dr and dp/dz inherit same exp(j m phi))

dPdr_2D = (dpdr(:) * ones(1,numel(theta_fig))) .* exp(1j*m_use*TH);

dPdz_2D = (dpdz(:) * ones(1,numel(theta_fig))) .* exp(1j*m_use*TH);

% ---------- cylindrical velocity components ----------

Vr = -(1j/(rho0*omega)) .* dPdr_2D; % v_r

Vz = -(1j/(rho0*omega)) .* dPdz_2D; % v_z

% v_phi: handle R=0 safely

Vphi = zeros(size(P_2D));

mask = (R > 0);

Vphi(mask) = (m_use./(rho0*omega*R(mask))) .* P_2D(mask); % v_phi

% ---------- convert to Cartesian on xOy ----------

Vx = Vr.*cos(TH) - Vphi.*sin(TH);

Vy = Vr.*sin(TH) + Vphi.*cos(TH);

% ---------- visualization 1: SPL background + instantaneous velocity arrows ----------

SPL_xy = 20*log10(abs(P_2D)/medium.pref/sqrt(2));

% choose a time snapshot (t=0): v_inst = Re{v}

Vx_t0 = real(Vx);

Vy_t0 = real(Vy);

% decimate for quiver (otherwise too dense)

skip_r = max(1, round(numel(rho_fig)/40));

skip_t = max(1, round(numel(theta_fig)/80));

Xq = X(1:skip_r:end, 1:skip_t:end);

Yq = Y(1:skip_r:end, 1:skip_t:end);

Uq = Vx_t0(1:skip_r:end, 1:skip_t:end);

Vq = Vy_t0(1:skip_r:end, 1:skip_t:end);

figure('Name',sprintf('Velocity field @ xOy, z=%.6f m (FHT)', z_use), 'position',[100 100 900 750]);

pcolor(X, Y, SPL_xy); shading flat;

colormap(MyColor('vik'));

clb = colorbar; clb.Title.Interpreter='latex';

clb.Title.String = '$\mathrm{SPL}$ (dB re $p_{\rm ref}$)';

set(clb,'Fontsize',18);

axis equal tight;

xlim([-r_boundary r_boundary]); ylim([-r_boundary r_boundary]);

set(gca,'linewidth',2,'TickLabelInterpreter','latex');

xlabel('$x$ (m)','Interpreter','latex','Fontsize',18);

ylabel('$y$ (m)','Interpreter','latex','Fontsize',18);

title(sprintf('$xOy$ @ $z=%.6f$ m, $m=%d$: SPL + $\\Re\\{\\mathbf v\\}$ arrows', z_use, m_use), ...

'Interpreter','latex','Fontsize',18);

hold on;

quiver(Xq, Yq, Uq, Vq, 'k'); % black arrows

hold off;

% ---------- visualization 2: |v_xy| magnitude (optional but recommended) ----------

Vxy_mag = sqrt(abs(Vx).^2 + abs(Vy).^2); % magnitude of complex transverse velocity

figure('Name',sprintf('|v_xy| @ xOy, z=%.6f m (FHT)', z_use), 'position',[100 100 900 750]);

pcolor(X, Y, Vxy_mag); shading flat;

colormap(MyColor('vik'));

clb = colorbar; clb.Title.Interpreter='latex';

clb.Title.String = '$|\mathbf v_{xy}|$ (m/s)';

set(clb,'Fontsize',18);

axis equal tight;

xlim([-r_boundary r_boundary]); ylim([-r_boundary r_boundary]);

set(gca,'linewidth',2,'TickLabelInterpreter','latex');

xlabel('$x$ (m)','Interpreter','latex','Fontsize',18);

ylabel('$y$ (m)','Interpreter','latex','Fontsize',18);

title(sprintf('$xOy$ @ $z=%.6f$ m, $m=%d$: $|\\mathbf v_{xy}|$', z_use, m_use), ...

'Interpreter','latex','Fontsize',18);

%% ==================== local functions ====================

function mem_mb = local_get_mem_mb()

% Returns MATLAB process memory usage in MB if possible; otherwise NaN.

mem_mb = NaN;

% Prefer Windows: use memory() for MATLAB process peak/usage

if ispc

try

m = memory();

mem_mb = double(m.MemUsedMATLAB) / (1024^2);

return;

catch

end

end

% macOS/Linux: Java heap usage (trend only, not full RSS)

try

rt = java.lang.Runtime.getRuntime();

used = double(rt.totalMemory() - rt.freeMemory()); % bytes

mem_mb = used / (1024^2);

catch

mem_mb = NaN;

end

end

function s = local_fmt_mem(x)

if isnan(x)

s = 'NaN';

else

s = sprintf('%.1f MB', x);

end

end